MedCare

Automated Medication Dispensing System

Product Requirements Document

Version 1.0  ·  March 2026

Saensuk Healthcare Nonthaburi

# 1. Executive Summary

MedCare is an automated pill dispensing system designed for elderly care facilities. It replaces the manual weekly medication sorting process — where nurses spend hours reading Med Sheets, counting pills by hand, and filling pill organizer boxes — with an automated, software-guided workflow that eliminates human error and frees nursing staff for direct patient care.

The core problem: At Saensuk Healthcare Nonthaburi (54 residents), caregivers currently sort medication manually once per week using paper Med Sheets. The process is error-prone: staff miss updates when doctors change prescriptions, pills are miscounted, and half-tablet cutting introduces inconsistencies. A single mistake can affect a resident's health for the entire week.

The solution: MedCare automates the weekly fill process. A nurse loads the correct total pill quantities into a Raspberry Pi-controlled dispenser, then the machine dispenses pills meal-by-meal into a collection cup. The caregiver transfers each batch into the corresponding slot in the resident's pill organizer box. The system is guided by a 7-inch touchscreen, connected to a Next.js web app for medication management and a FastAPI + Supabase backend.

Impact Reduces weekly medication sorting time by an estimated 60–70%, eliminates common counting errors, and creates a digital audit trail for every dispense event.

# 2. Background & Field Research

## 2.1 Site Research

Field research was conducted at Saensuk Healthcare Nonthaburi through staff interviews and observation sessions. Key participants included the head nurse (Khun Bus) and caregiver Khun May, who is responsible for 8 residents.

| Facility Attribute | Details |
| --- | --- |
| Total residents | 54 |
| Nursing staff | 1 head nurse (Khun Bus) |
| Caregiver ratio | 1 caregiver : 4–5 residents |
| Medication frequency | Up to 4 times/day (before breakfast, after breakfast, after dinner, bedtime) |
| Maximum meds per resident | 20+ tablets/day for some residents |
| Current sorting method | Manual, once per week using paper Med Sheets |
| Pill organizer format | Plastic weekly organizer boxes with labeled day/meal slots |

## 2.2 Identified Pain Points

The following issues were identified through direct observation and staff interviews:

| # | Pain Point | Description |
| --- | --- | --- |
| 1 | Stale Med Sheets | Paper Med Sheets are not updated in real-time. When a doctor changes a prescription, staff may not see the update before the next weekly sort. |
| 2 | Human counting errors | Staff manually count dozens of tablets per resident without a double-check mechanism. Errors are only discovered when a resident receives the wrong dose. |
| 3 | Half-tablet and crushed meds | Some residents require half-tablets or fully crushed medication (mixed into food). Cutting and crushing by hand is time-consuming and imprecise. |
| 4 | No stock visibility | There is no system to track when a medication is running low. Stock shortages are discovered only at the moment of dispensing. |
| 5 | Shift handover risk | Knowledge of each resident's medication needs is not formally transferred during shift changes, creating risk when new staff are assigned. |

# 3. Product Overview

## 3.1 System Architecture

MedCare consists of three integrated components:

| Component | Technology | Role |
| --- | --- | --- |
| Dispenser Machine | Raspberry Pi 4 + Moonraker API + Stepper Motors | Physical pill dispensing unit. Receives dispense commands from the backend via Moonraker API and releases pills into collection cups. |
| Touchscreen UI | HTML/CSS/JS on 7" display (landscape, 1024×600) | Guides the nurse through the weekly fill workflow. Shows which medications to load, then triggers dispense meal-by-meal. |
| Web Application | Next.js (frontend) + Elysia.js + FastAPI + Supabase | Manages resident profiles, medication schedules, stock levels, and dispense history. Syncs with the dispenser. |

## 3.2 Core User Workflow

The primary use case is the weekly medication fill session, completed once per week per resident:

- Nurse selects a resident from the touchscreen.

- System displays the medication loading list — each drug with the total tablet count needed for the full week.

- Nurse physically loads tablets into the designated compartments of the dispenser machine.

- Nurse taps "Start Dispense."

- Machine dispenses pills meal-by-meal into a collection cup (e.g., Before Breakfast pills for all 7 days).

- Nurse transfers pills from the cup into the correct slots of the resident's pill organizer box, then taps "Done."

- Machine automatically advances to the next meal. Steps 5–6 repeat until all meals are complete.

- Process repeats for the next resident.

Key Insight The machine dispenses all 7 days' worth of one meal at once — the nurse places them into 7 daily slots in one motion. This mirrors the existing organizer box format used at the facility.

# 4. Users & Roles

| Role | Primary Interface | Responsibilities |
| --- | --- | --- |
| Head Nurse | Web App + Touchscreen | Manages medication schedules, edits resident drug profiles, reviews dispense history and stock alerts. |
| Caregiver | Touchscreen | Loads medication into the machine, operates the weekly fill workflow, confirms each meal batch. |
| Family Member | Web App (read-only) | Views their relative's medication schedule and dispense records. |
| Administrator | Web App | Manages user accounts, facility settings, and system configuration. |

# 5. Feature Requirements

## 5.1 Touchscreen Application (MVP)

The touchscreen UI operates in 4 sequential steps per resident:

### Step 1 — Select Resident

- Display all residents in a grid, grouped by status (pending / completed).

- Show weekly progress counter (e.g., "6 of 54 done").

- Highlight urgent residents (e.g., low stock warning) with a red indicator.

- Cards for completed residents are greyed out and non-interactive.

### Step 2 — Medication Loading List

- Display each medication with: drug name, dosage, special instructions, and total tablet count needed for the week.

- Show which meals each medication belongs to (color-coded chips).

- Nurse taps a checkmark (✓) per medication after physically loading it.

- Special flags: crushed medication, liquid/eye drops, timed doses.

- Low-stock warning shown inline if any medication quantity is critically low.

- "Start Dispense" button activates only when all medications are confirmed loaded (all checkmarks ticked).

### Step 3 — Dispense Per Meal

- Left sidebar shows all meals for the day with status (pending / active / done).

- Current meal shown prominently with icon, name, and item count.

- Each medication within the meal is animated as it is dispensed (idle → dispensing → done).

- Tray indicator shows pills accumulating with running count.

- "Transfer to box" instruction appears when all pills in the batch are dispensed, showing which slot to fill (e.g., "Before Breakfast").

- Nurse confirms transfer → machine moves to next meal automatically.

### Step 4 — Completion Summary

- Display total pills dispensed, number of medications, and days covered.

- Two action buttons: "Return Home" (mark current resident done) and "Next Resident."

## 5.2 Web Application (MVP)

### Resident Management

- Create and edit resident profiles: name, room number, ward, allergies, chronic conditions, doctor contact.

- Attach a photo to each resident profile.

- Set special dispensing flags: requires crushing, liquid medication, requires assistance.

### Medication Schedule

- Digital Med Sheet: list of all active medications per resident with dosage, frequency, and meal assignments.

- Real-time sync: changes made by the head nurse are immediately reflected on the touchscreen.

- Change highlighting: medications added or modified in the last 48 hours are flagged visually.

- Drug interaction detection: warn if two medications are known to interact (Phase 2).

### Stock Tracking

- Track current quantity for each medication in inventory.

- Automatically calculate estimated run-out date based on daily consumption.

- Alert thresholds: Warning at 14 days remaining, Critical at 7 days remaining.

- Notify family members via LINE when a medication stock is critically low.

- Maintain a request log: track whether family has been notified, acknowledged, and when medication was received.

### Dispense History

- Log every dispense event: resident, medications dispensed, meal, date, time, and which staff member initiated.

- View dispense history per resident and per date.

- Export dispense records (PDF) for compliance documentation.

### Shift Handover

- Auto-generate a shift summary: residents with upcoming dispense needs, stock alerts, and any flagged medication changes.

- Nurse reviews and signs off on handover before ending shift.

# 6. Non-Functional Requirements

| Category | Requirement |
| --- | --- |
| Reliability | Dispenser machine must complete a dispense cycle without failure ≥ 99.5% of the time under normal operating conditions. |
| Offline Capability | Touchscreen application must function without internet connectivity. Dispense operations use local data cache; sync with backend when connectivity is restored. |
| Latency | Touchscreen UI must respond to touch input within 200ms. Dispense command must be received by the machine within 1 second of confirmation tap. |
| Accessibility | All touchscreen text must be ≥ 16px (preferably ≥ 18px) for readability in clinical lighting. Buttons must be ≥ 48×48px for reliable touch. |
| Security | Resident health data must be stored encrypted at rest. Access to the web application requires role-based authentication. API endpoints require JWT tokens. |
| Audit Trail | Every dispense event must be logged with a timestamp, staff ID, and outcome. Logs must be immutable and retained for a minimum of 2 years. |
| Language | All user-facing interfaces must support Thai language. Technical logs and API responses may be in English. |

# 7. Technical Architecture

## 7.1 Technology Stack

| Layer | Technology | Notes |
| --- | --- | --- |
| Frontend (Web) | Next.js + TypeScript + Tailwind CSS | Deployed on Vercel or self-hosted. SSR for fast initial load. |
| Frontend (Touchscreen) | HTML / CSS / JS (vanilla) | Runs in a kiosk browser on the Raspberry Pi touchscreen. No framework dependency to minimize startup time. |
| API Gateway | Elysia.js (Bun runtime) | High-performance REST API. Handles authentication, routing, and business logic. |
| ML / Inference | FastAPI (Python) | Hosts OCR model for prescription scanning (Phase 2) and any ML-based drug interaction checks. |
| Database | Supabase (PostgreSQL + Realtime) | Primary data store. Realtime subscriptions used for live Med Sheet sync between web app and touchscreen. |
| Hardware Control | Raspberry Pi 4 + Moonraker API | Moonraker acts as the motion control layer. Dispense commands translate to stepper motor positions. |
| Notifications | LINE Messaging API | Push alerts to family members for stock warnings and weekly dispense summaries. |
| Auth | Supabase Auth (JWT) | Role-based: admin, nurse, caregiver, family (read-only). |

## 7.2 Hardware Components

- Raspberry Pi 4 (4GB RAM) — primary compute unit

- 7-inch DSI touchscreen display (1024×600, landscape mount)

- Stepper motor array — one motor per medication channel

- Vacuum gripper / pill channel mechanism — controls pill release

- LED guidance strip — illuminates active medication channel

- Collection cup sensor — confirms pills have been dispensed

- Wi-Fi module — connects to facility network for backend sync

# 8. Scope & Phasing

## Phase 1 — MVP (Target: Q2 2026)

- Touchscreen UI: full 4-step weekly fill workflow

- Web App: resident profiles, digital Med Sheet, basic stock tracking

- Hardware: single-channel prototype dispenser (proof of concept)

- Backend: Elysia.js API, Supabase database, Raspberry Pi + Moonraker integration

- Notifications: LINE alerts for stock warnings

## Phase 2 — Enhanced Automation (Q3–Q4 2026)

- OCR prescription scanning: scan medication packaging to auto-populate Med Sheet

- Multi-channel dispenser: parallel pill channels for faster filling

- Drug interaction detection: warn on potentially harmful combinations

- Shift handover module: auto-generated handover summaries

- Family portal: web dashboard for relatives to view resident medication status

## Phase 3 — Scale & Compliance (2027)

- Multi-facility support: manage multiple care homes from one platform

- Regulatory compliance reporting: auto-generated reports for Thai Ministry of Public Health standards

- Predictive stock ordering: AI-suggested reorder quantities and timing

# 9. Success Metrics

| Metric | Baseline (Current) | Target (Post-Launch) |
| --- | --- | --- |
| Weekly medication sort time (per resident) | ~8–12 minutes | < 3 minutes |
| Medication dispensing errors (per 100 fills) | Unknown (untracked) | < 1 error |
| Stock-out incidents (per month) | ~2–3 incidents | 0 surprise stock-outs |
| Staff time spent on Med Sheet updates | Manual, per shift | Real-time, automated |
| Audit trail availability | None | 100% of dispense events logged |
| Caregiver task completion rate | Untracked | > 95% weekly fills completed on schedule |

# 10. Risks & Mitigations

| # | Risk | Severity | Mitigation |
| --- | --- | --- | --- |
| R1 | Mechanical jam during dispense | High | Sensor detects blockage and halts operation. Manual override mode allows nurse to complete fill by hand. Alert sent to admin. |
| R2 | Incorrect pill count due to pill size variation | High | Calibration mode per medication type. Weight sensor validates dispense count. Nurse performs visual confirmation before transfer. |
| R3 | Network outage during operation | Medium | Offline-first architecture. Touchscreen caches resident and medication data locally. Sync on reconnect. |
| R4 | Staff resistance to adoption | Medium | Touchscreen UX optimized for minimal training. Large buttons, Thai language, visual step indicators. Pilot with 1–2 caregivers before full rollout. |
| R5 | Regulatory compliance for automated dispensing | Medium | System aids preparation but final dispensing to resident is still performed by trained caregiver. Consult Thai FDA guidelines during Phase 1. |
| R6 | Liquid medications and eye drops | Low | Phase 1 scope excludes liquid medications. System flags these for manual preparation with on-screen instructions. |

# 11. Open Questions

- What is the maximum number of distinct medication channels the physical dispenser will support in Phase 1? (Currently assumed: 6–8 channels per unit)

- What are the Thai FDA requirements for automated dispensing devices in licensed elderly care facilities?

- Should the system support partial-week fills (e.g., resident starts a new medication mid-week)?

- What is the Wi-Fi infrastructure at the facility? Is a dedicated VLAN needed for the Raspberry Pi units?

- Will family members use LINE or a separate web portal for notifications and status viewing?

- How should the system handle medications that require refrigeration or special storage?

# 12. Appendix

## A. Glossary

| Term | Definition |
| --- | --- |
| Med Sheet | The paper (or digital) document listing each resident's active medications, dosages, and meal schedule. The source of truth for medication preparation. |
| Pill Organizer Box | A plastic box with 28 compartments (7 days × 4 meals) used to pre-sort a week's medication for one resident. |
| Dispense Event | A single completed cycle of the machine dispensing pills for one meal into the collection cup. |
| Weekly Fill | The complete process of loading all medications for one resident and dispensing them across all meals for all 7 days. |
| Moonraker API | Open-source 3D printer motion control API running on Raspberry Pi, repurposed here to control stepper motors for pill dispensing. |
| Double Check | A verification step where the caregiver confirms the loaded medication list before dispensing begins. |

## B. Reference Documents

- Field Research Notes — Saensuk Healthcare Nonthaburi (March 2026)

- MedCare System Architecture Diagram (v1.0)

- Touchscreen UI Prototype — medcare-dispenser-touch.html

- Mobile App UI Prototypes — Dashboard, Patient Profile, Dispense Checklist, Stock Management

- Moonraker API Documentation — moonraker.readthedocs.io